2025/08/21 Log of CTR Experiment

Previous 6/6 Slit condition

- I0 slit H : 0.10 mm

- I0 slit W : 5.00 mm

- R1 slit H : 0.10 mm

- R1 slit W : 5.00 mm

- R2 slit H : 0.10 mm

- R2 slit W : 5.00 mm

Today’s slit condition

- I0 slit H : 0.20 mm

- I0 slit W : 1.00 mm

- R1 slit H : 0.20 mm

- R1 slit W : 1.00 mm

- R2 slit H : 0.20 mm

- R2 slit W : 1.00 mm

I0 slit W is focused (reduced) in order to decrease the sample inhomogeneity.

I0 slit H becomes larger in order to increase the signal intensity.

—--------------------------------------------------------------

10:12 Sample: Si_Thermo

ω-scan ±0.1 deg Δω=0.001 deg

File Name : Si_Thermo_omega_001.xlsx (original data:Si_Thermo_omega.dat)

10:36 θ-2θ-scan

69.5910 ±1 deg Δ=0.01 deg 1sec/point

File Name:Si_Thermo_th2th_001.xlsx (original data: Si_Thermo_2thome.dat)

11:09 TEOS試料に変更

11:32 ω-scan ±0.1 deg Δω=0.001 deg

File Name : Si_TEOS_omega_001.xlsx (original data:Si_TEOS_omega.dat)

11:36 θ-2θ-scan

69.5910 ±1 deg Δ=0.01 deg 1sec/point

File Name:Si_TEOS_th2th_001.xlsx (original data: Si_TEOS_2thome.dat)

12:10 Rough TEOS試料に変更

12:29 ω-scan ±0.1 deg Δω=0.001 deg

File Name : Si_roughTEOS_omega_001.xlsx (original data:Si_roughTEOS_omega.dat)

12:40 θ-2θ-scan

69.5910 ±1 deg Δ=0.01 deg 1sec/point

File Name:Si_roughTEOS_th2th_001.xlsx (original data: Si_roughTEOS_2thome.dat)

13:05 ω-scan ±0.1 deg Δω=0.001 deg

File Name : Si_roughTEOS_omega_001.xlsx (original data:Si_roughTEOS_omega_BG.dat)

Rough TEOS試料を90deg回転させる(反時計回り)

14:29 ω-scan ±0.1 deg Δω=0.001 deg

File Name : Si_roughTEOS_omega_002.xlsx (original data:Si_roughTEOS_omega2.dat)

14:40 θ-2θ-scan

69.5910 ±1 deg Δ=0.01 deg 1sec/point

File Name:Si_roughTEOS_th2th_001.xlsx (original data: Si_roughTEOS_2thome_2.dat)

14:47 ω-scan ±0.1 deg Δω=0.001 deg

File Name : Si_roughTEOS_omega_002.xlsx (original data:Si_roughTEOS_omega_2_BG.dat)

TEOS試料を90deg回転させる(反時計回り)

15: ??ω-scan ±0.1 deg Δω=0.001 deg

File Name : Si_TEOS_omega_002.xlsx (original data:Si_TEOS_omega2.dat)データ保存にミス。

15:40 θ-2θ-scan

69.5910 ±1 deg Δ=0.01 deg 1sec/point

File Name:Si_TEOS_th2th_002.xlsx (original data: Si_TEOS_2thome_2.dat)

16:02 ω-scan

69.5910 ±1 deg Δω=0.001 deg

File Name : Si_TEOS_omega_2.xlsx (original data: Si_TEOS_omega_2.dat)

16:14 θ-2θ-scan

69.5910 ±1 deg Δ=0.01 deg 1sec/point

File Name : Si_TEOS_2thome_2.xlsx (original data:Si_TEOS_2thome_2.dat)

16:18  ω-scan

69.5910 ±1 deg Δω=0.0005 deg

File Name : Si_TEOS_omega_2_BG.xlsx (original data:Si_TEOS_omega_2_BG.dat)

17:28  θ-2θ-scan

69.5910 ±1 deg Δ=0.01 deg 1sec/point

File Name : Si_Thermo_2thome_2.xlsx (original data:Si_Thermo_2thome_2.dat)

17:42  ω-scan

69.5910 ±1 deg Δω=0.001 deg

File Name : Si_Thermo_omega_2.xlsx (original data:Si_Thermo_omega_2.dat)

17:50  ω-scan

69.5910 ±1 deg Δω=0.0005 deg

File Name : Si_Thermo_omega_2_BG.xlsx (original data:Si_Thermo_omega_2_BG.dat)

—--------------------------------------------

2025/08/22

カメラ長　709.0218±0.008mm

カメラ受光面中心位置　(x,y)=(504.5, 251.5) pixel

10:30  ω-scan

35.1065 ±0.5 deg Δω=0.01deg

File Name : Thermo_3_2D-omega

10:50  ω-scan

35.1065 ±0.5 deg Δω=0.01deg

File Name : Thermo_4_2D-omega

12:04  ω-scan

35.1065 ±0.5 deg Δω=0.01deg

File Name : TEOS_3_2D-omega

12:21 (004) → (220) for TEOS

12:44  ω-scan

23.2955 ±0.5 deg Δω=0.01deg

File Name : TEOS_220_2D-omega

14:26 Change sample RoughTEOS

KOHZU XA07A-R103

駿河精機 KXTシリーズ

70□ X-stage Thickness 20mm

https://jpn.surugaseiki.com/products/category/A01_x-axis-motorized-stage

https://jpn.surugaseiki.com/products/products?katashiki=PMG715M-LA-C

14:36  ω-scan

34.8948 ±0.5 deg Δω=0.01deg

File Name : RoughTEOS_3_2D-omega

15:24  ω-scan

23.7999 ±0.5 deg Δω=0.01deg

File Name : Thermo_220_2D-omega

RoI (Region of Interest) 280pixel square